esphome:

name: "saltowl"

friendly_name: Salt Owl

esp8266:

board: esp01_1m

# Enable logging

logger:

# Enable Home Assistant API

api:

encryption:

key: "1hRF7mhHjtpM+rYixJKrhy9oE9fgeexbqZdtyb1UHIs="

ota:

- platform: esphome

password: "a95dafab2d8c84051f2cdcbebd0c28cf"

wifi:

ssid: !secret wifi_ssid

password: !secret wifi_password

manual_ip:

static_ip: 192.168.11.166

gateway: 192.168.11.1

subnet: 255.255.255.0

# Enable fallback hotspot (captive portal) in case wifi connection fails

ap:

ssid: "Saltyowl-2 Fallback Hotspot"

password: "EPVMmq21Ipzm"

captive_portal:

# --- Sensors: raw ultrasonic distance readings ---

sensor:

- platform: ultrasonic

trigger_pin: GPIO12

echo_pin: GPIO14

name: "Left Chamber"

id: left_distance

update_interval: 10s

unit_of_measurement: "cm"

filters:

- multiply: 100.0   # convert m → cm

- platform: ultrasonic

trigger_pin: GPIO1

echo_pin: GPIO3

name: "Right Chamber"

id: right_distance

update_interval: 10s

unit_of_measurement: "cm"

filters:

- multiply: 100.0   # convert m → cm

# --- Computed % full for Left chamber ---

- platform: template

name: "Left Chamber % Full"

id: left_percent

unit_of_measurement: "%"

accuracy_decimals: 0

update_interval: 2s

lambda: |-

if (!id(left_distance).has_state()) return NAN;

float dist = id(left_distance).state;

float full = id(left_full).state;

float empty = id(left_empty).state;

// Clamp

dist = clamp(dist, full, empty);

return 100.0 * (empty - dist) / (empty - full);

# --- Computed % full for Right chamber ---

- platform: template

name: "Right Chamber % Full"

id: right_percent

unit_of_measurement: "%"

accuracy_decimals: 0

update_interval: 2s

lambda: |-

if (!id(right_distance).has_state()) return NAN;

float dist = id(right_distance).state;

float full = id(right_full).state;

float empty = id(right_empty).state;

// Clamp

dist = clamp(dist, full, empty);

return 100.0 * (empty - dist) / (empty - full);

# --- Configurable calibration values (via HA UI) ---

number:

- platform: template

name: "SaltOwl Left Full Distance"

id: left_full

optimistic: true

min_value: 0

max_value: 100

step: 0.5

unit_of_measurement: "cm"

initial_value: 10.0

- platform: template

name: "SaltOwl Left Empty Distance"

id: left_empty

optimistic: true

min_value: 0

max_value: 100

step: 0.5

unit_of_measurement: "cm"

initial_value: 30.0

- platform: template

name: "SaltOwl Right Full Distance"

id: right_full

optimistic: true

min_value: 0

max_value: 100

step: 0.5

unit_of_measurement: "cm"

initial_value: 10.0

- platform: template

name: "SaltOwl Right Empty Distance"

id: right_empty

optimistic: true

min_value: 0

max_value: 100

step: 0.5

unit_of_measurement: "cm"

initial_value: 30.0

V2 code

Taken out

# --- % Full Left Chamber (bottom-up logic) ---

- platform: template

name: "Left Chamber % Full"

id: left_percent

internal: true

unit_of_measurement: "%"

accuracy_decimals: 0

update_interval: 2s

lambda: |-

if (!id(left_distance).has_state()) return NAN;

float dist = id(left_distance).state;

float fill_top = id(left_fill_top).state;

float max_height = id(left_max_height).state;

float bottom = fill_top + max_height;

dist = clamp(dist, fill_top, bottom);

return 100.0 * (bottom - dist) / max_height;

# --- % Full Right Chamber (bottom-up logic) ---

- platform: template

name: "Right Chamber % Full"

id: right_percent

internal: true

unit_of_measurement: "%"

accuracy_decimals: 0

update_interval: 2s

lambda: |-

if (!id(right_distance).has_state()) return NAN;

float dist = id(right_distance).state;

float fill_top = id(right_fill_top).state;

float max_height = id(right_max_height).state;

float bottom = fill_top + max_height;

dist = clamp(dist, fill_top, bottom);

return 100.0 * (bottom - dist) / max_height;

number:

# --- Left Chamber Calibration ---

- platform: template

name: "SaltOwl Left Fill Top (cm from sensor)"

id: left_fill_top

optimistic: true

internal: true

min_value: 0

max_value: 100

step: 0.5

unit_of_measurement: "cm"

initial_value: 10.0

- platform: template

name: "SaltOwl Left Max Height (cm)"

id: left_max_height

optimistic: true

internal: true

min_value: 1

max_value: 100

step: 0.5

unit_of_measurement: "cm"

initial_value: 20.0

# --- Right Chamber Calibration ---

- platform: template

name: "SaltOwl Right Fill Top (cm from sensor)"

id: right_fill_top

optimistic: true

internal: true

min_value: 0

max_value: 100

step: 0.5

unit_of_measurement: "cm"

initial_value: 10.0

- platform: template

name: "SaltOwl Right Max Height (cm)"

id: right_max_height

optimistic: true

internal: true

min_value: 1

max_value: 100

step: 0.5

unit_of_measurement: "cm"

initial_value: 20.0

🎉 Awesome! Glad to hear it's compiling cleanly now — that’s a solid milestone. You're now running a nicely structured ESPHome setup with:

- 🧠 Calibration variables (offset, depth, threshold) stored and synced with HA

- 📡 Ultrasonic distance measurements

- 📈 Fill % computed immediately after sensor update

- 🚨 Alerts for low fill in each chamber and average

- ✅ Resilience across reboots thanks to restore_value and on_boot

If you’re at the testing stage, here are a few optional next steps you might consider:

🧪 Dev/Test Ideas:

- Temporarily reduce update_interval to something like 10s while testing

- Add log_level: DEBUG to your logger: block to see real-time values

- Use a text_sensor: to display the average fill in one spot

- Add status_led if your board supports it for visual state

🛡️ Stability Tips:

- Monitor how quickly HA resyncs values after a reboot

- Validate alert triggers by simulating low fill levels

- Keep an eye on the logs for noise or unexpected jumps in sensor readings

hmm, log spam....tell me more

ChatGPT said:

Ah yes — log spam in ESPHome can become a real pain, especially when:

- You're polling sensors frequently (even at 5s or 10s)

- You have template or copy sensors that report the same value repeatedly

- You’re trying to monitor the logs for real issues, but they get drowned out by constant updates

🧾 Typical Symptoms of Log Spam

You’ll see stuff like this every few seconds:

[11:02:05][D][sensor:127]: 'Left Chamber Sensor Distance': Sending state 63.45000 cm

[11:02:05][D][sensor:127]: 'Right Chamber Sensor Distance': Sending state 63.40000 cm

[11:02:05][D][sensor:127]: 'Left Chamber Fill (%)': Sending state 58.0 %

[11:02:05][D][sensor:127]: 'Right Chamber Fill (%)': Sending state 57.0 %

[11:02:05][D][binary_sensor:036]: 'Left Chamber Fill Low': Sending state OFF

[11:02:05][D][binary_sensor:036]: 'Right Chamber Fill Low': Sending state OFF

If values don’t change much — or at all — these logs just fill up your console, serial monitor, or OTA log view.

✅ Strategies to Reduce Log Spam

1. Use filters: like delta or heartbeat

In sensor: components, you can add filters like:

filters:

- delta: 1.0   # Only report if change > 1.0

- heartbeat: 60s  # Always report at least every 60s

🔹 Example:

- platform: ultrasonic

trigger_pin: GPIO12

echo_pin: GPIO14

name: "Left Chamber Sensor Distance"

id: left_distance

update_interval: 10s

unit_of_measurement: "cm"

filters:

- multiply: 100.0

- delta: 1.0

- heartbeat: 60s

This way:

- You only log when something actually changes

- You still get occasional updates (every 60s) even if nothing changes

2. Use internal: true for sensors you don't need in HA

You're already doing this for left_percent and right_percent — nice!
This means those internal template sensors won’t log or publish unless you explicitly expose them.

✅ Avoid exposing noisy sensors unless absolutely needed in HA.

3. Disable logs from specific components

You can’t turn off logs per sensor yet, but you can control the log level globally:

logger:

level: WARN

Valid levels:

- NONE

- ERROR

- WARN

- INFO (default)

- DEBUG

- VERBOSE

- VERY_VERBOSE

So if you're not debugging, switching to WARN will cut out most routine sensor logs.

4. Avoid update_interval: on template: sensors if you're manually updating

Since you’re already triggering % full updates via on_value, make sure you don’t have update_interval: on the template sensor. Otherwise, it still runs periodically and spams logs.

✅ Your current setup already does this — good!

5. Use force_update: false to suppress duplicate logs

If a sensor reports the same value as last time, ESPHome won't log it unless:

- force_update: true is set

So make sure you're not forcing updates unnecessarily:

sensor:

- platform: template

...

force_update: false  # <- default, can be omitted

🔇 TL;DR — Reduce Log Spam by:

| What | Why |
| --- | --- |
| filters: delta and heartbeat | Only log real changes |
| internal: true | Silence non-HA-facing sensors |
| logger.level: WARN | Reduce overall log verbosity |
| Avoid update_interval: on manually updated sensors | Prevent double-processing |
| force_update: false | Avoid duplicate state logs |

If you'd like, I can walk you through tailoring this to your current setup — or give you a “quiet mode” config template. Let me know how minimal you want the logs.